(IS AC/DC)

AC/DC RELAY

The IS AC/DC relay is a very versatile relay as it will operate on AC voltages ranging from 8 volts to 32 volts, and it will also operate on DC voltages ranging from 10 volts to 35 volts.

The double pole relay is a simple to fit module wherever a dry contact, voltage free switching is required or where there is a need for higher than usual currents and voltages to be switched. The double pole relay is rated at 8 Amps complete with LED indication.

- Works on AC or DC

- Automatic voltage selection

- No jumper links to select voltage

- Not polarity conscious.

- Energy saving feature that allows the relay to work at higher voltages

| Supply Voltage | 6 - 32 Volts AC 10 – 35 Volts AC |
| --- | --- |
| Coil Resistance | 360 Ohms |
| Nominal Current | 20 mA |
| Double Pole Relay rated at | 8 Amps at 240 V AC 8 Amps at 30 V DC |
| Minimum switching current | 10 mA (to keep contacts clean) |
| Dimensions | 56mm (W) 69mm (L) 30mm (H) |

We recommend IS Sticky Feet Size B,

self-adhesive mounting pillars to ensure

a neat fixing.